## Safety Data Sheet

### META

Document Nbr: 082610

Class C Parts IDs BD1132,BD1132-1

SAFETY DATA SHEET

ISSUING DATE: 16-APR-2015

REVISION DATE: 13-JUN-2015

VERSION: 2.02

------1. IDENTIFICATION OF THE SUBSTANCE/PREPARATION ------

AND OF THE COMPANY/UNDERTAKING

PRODUCT IDENTIFIER:

PRODUCT NAME: 64707763 BD1132 ANTI-RUST META

RECOMMENDED USE OF THE CHEMICAL AND RESTRICTIONS ON USE:

PRODUCT CODE: F01555

PRODUCT TYPE: EXTREMELY FLAMMABLE AEROSOL

SYNONYMS: NONE

SUPPLIER'S DETAILS:

RECOMMENDED USE: METAL PROTECTANT.

USES ADVISED AGAINST: NO INFORMATION AVAILABLE

MANUFACTURER:

AMERICAN JETWAY CORPORATION

34136 MYRTLE STREET

WAYNE, MI 48184-0126

DISTRIBUTOR:

CLASS C SOLUTIONS GROUP

A BUSINESS OF MSC INDUSTRIAL SUPPLY CO.

75 MAXESS ROAD

MELVILLE, NY 11747-3151

1-866-438-6767

EMERGENCY TELEPHONE NUMBER:

CHEMICAL EMERGENCY PHONE NUMBER:

CHEMTREC: 1-800-262-8200 ID 1195

COMPANY EMERGENCY PHONE NUMBER: 734-721-5930

------2. HAZARDS IDENTIFICATION ------

CLASSIFICATION:

SKIN CORROSION/IRRITATION: CATEGORY 2

SERIOUS EYE DAMAGE/EYE IRRITATION: CATEGORY 2B

REPRODUCTIVE TOXICITY: CATEGORY 2

SPECIFIC TARGET ORGAN TOXICITY (SINGLE EXPOSURE): CATEGORY 3

SPECIFIC TARGET ORGAN TOXICITY (REPEATED EXPOSURE): CATEGORY 1

ASPIRATION TOXICITY: CATEGORY 1

FLAMMABLE AEROSOLS: CATEGORY 1

GHS LABEL ELEMENTS, INCLUDING PRECAUTIONARY STATEMENTS:

EMERGENCY OVERVIEW:

DANGER:

HAZARD STATEMENTS:

CAUSES SKIN IRRITATION

CAUSES EYE IRRITATION

SUSPECTED OF DAMAGING FERTILITY OR THE UNBORN CHILD

MAY CAUSE RESPIRATORY IRRITATION. MAY CAUSE DROWSINESS OR DIZZINESS

MAY CAUSE DAMAGE TO ORGANS (CENTRAL NERVOUS SYSTEM, EYES, KIDNEY,

RESPIRATORY SYSTEM, AND SKIN) THROUGH PROLONGED OR REPEATED EXPOSURE.

MAY BE FATAL IF SWALLOWED AND ENTERS AIRWAYS

EXTREMELY FLAMMABLE AEROSOL

EXCLAMATION MARK

HEALTH HAZARD

FLAME

APPEARANCE: OPAQUE

PHYSICAL STATE: AEROSOL

ODOR: SOLVENT

PRECAUTIONARY STATEMENTS - PREVENTION:

OBTAIN SPECIAL INSTRUCTIONS BEFORE USE

DO NOT HANDLE UNTIL ALL SAFETY PRECAUTIONS HAVE BEEN READ AND UNDERSTOOD

USE PERSONAL PROTECTIVE EQUIPMENT AS REQUIRED

WASH FACE, HANDS AND ANY EXPOSED SKIN THOROUGHLY AFTER HANDLING

DO NOT BREATHE DUST/FUME/GAS/MIST/VAPORS/SPRAY

DO NOT EAT, DRINK OR SMOKE WHEN USING THIS PRODUCT

USE ONLY OUTDOORS OR IN A WELL-VENTILATED AREA

KEEP AWAY FROM HEAT/SPARKS/OPEN FLAMES/HOT SURFACES. - NO SMOKING

DO NOT SPRAY ON AN OPEN FLAME OR OTHER IGNITION SOURCE

PRESSURIZED CONTAINER: DO NOT PIERCE OR BURN, EVEN AFTER USE

PRECAUTIONARY STATEMENTS - RESPONSE:

IF EXPOSED OR CONCERNED:

GET MEDICAL ADVICE/ATTENTION SPECIFIC TREATMENT (SEE FIRST AID ON THIS

LABEL)

IF IN EYES:

RINSE CAUTIOUSLY WITH WATER FOR SEVERAL MINUTES. REMOVE CONTACT LENSES, IF

PRESENT AND EASY TO DO. CONTINUE RINSING.

IF EYE IRRITATION PERSISTS: GET MEDICAL ADVICE/ATTENTION

IF ON SKIN: WASH WITH PLENTY OF SOAP AND WATER.

IF SKIN IRRITATION OCCURS:

GET MEDICAL ADVICE/ATTENTION TAKE OFF CONTAMINATED CLOTHING AND WASH BEFORE

REUSE

IF INHALED:

REMOVE PERSON TO FRESH AIR AND KEEP COMFORTABLE FOR BREATHING.

CALL A POISON CENTER OR DOCTOR/PHYSICIAN IF YOU FEEL UNWELL

IF SWALLOWED:

IMMEDIATELY CALL A POISON CENTER OR DOCTOR/PHYSICIAN DO NOT INDUCE VOMITING

PRECAUTIONARY STATEMENTS - STORAGE:

STORE LOCKED UP

STORE IN A WELL-VENTILATED PLACE. KEEP CONTAINER TIGHTLY CLOSED

PROTECT FROM SUNLIGHT. DO NOT EXPOSE TO TEMPERATURES EXCEEDING

50 DEG. C/122 DEG. F

PRECAUTIONARY STATEMENTS - DISPOSAL:

DISPOSE OF CONTENTS/CONTAINER TO AN APPROVED WASTE DISPOSAL PLANT

HAZARDS NOT OTHERWISE CLASSIFIED (HNOC): NONE

OTHER INFORMATION:

0% OF THE MIXTURE CONSISTS OF INGREDIENT(S) OF UNKNOWN TOXICITY

------3. COMPOSITION/INFORMATION ON INGREDIENTS ------

CAS # 64742-49-0, COMMERCIAL HEXANES, MAY BE SUBSTITUTED FOR CAS #110-54-3.

CHEMICAL NAME                         CAS-NO           WEIGHT %*

HEXANE                                64742-49-0       40-50

PROPANE/ISOBUTANE/N-BUTANE            68476-86-8       20-30

PETROLEUM DISTILLATES                  8052-41-3       10-20

*THE EXACT PERCENTAGE (CONCENTRATION) OF COMPOSITION HAS BEEN WITHHELD AS

A TRADE SECRET.

------4. FIRST AID MEASURES ------

FIRST AID MEASURES FOR DIFFERENT EXPOSURE ROUTES:

EYE CONTACT:

RINSE IMMEDIATELY WITH PLENTY OF WATER, ALSO UNDER THE EYELIDS, FOR AT

LEAST 15 MINUTES. KEEP EYE WIDE OPEN WHILE RINSING. IF SYMPTOMS PERSIST,

CALL A PHYSICIAN.

SKIN CONTACT:

WASH OFF IMMEDIATELY WITH SOAP AND PLENTY OF WATER WHILE REMOVING ALL

CONTAMINATED CLOTHES AND SHOES. IF SYMPTOMS PERSIST, CALL A PHYSICIAN.

INHALATION:

MOVE TO FRESH AIR. IF BREATHING IS DIFFICULT, GIVE OXYGEN. ARTIFICIAL

RESPIRATION AND/OR OXYGEN MAY BE NECESSARY. IF BREATHING HAS STOPPED,

CONTACT EMERGENCY MEDICAL SERVICES IMMEDIATELY.

INGESTION:

RINSE MOUTH. DO NOT INDUCE VOMITING. NEVER GIVE ANYTHING BY MOUTH TO AN

UNCONSCIOUS PERSON. CALL A PHYSICIAN OR POISON CONTROL CENTER IMMEDIATELY.

MOST IMPORTANT SYMPTOMS/EFFECTS, ACUTE AND DELAYED:

MAIN SYMPTOMS:

IRRITATING TO SKIN. CAUSES SERIOUS EYE DAMAGE. CAUSES DROWSINESS OR

DIZZINESS. HARMFUL OR FATAL IF SWALLOWED AND ENTERS AIRWAYS.

INDICATION OF IMMEDIATE MEDICAL ATTENTION AND SPECIAL TREATMENT NEEDED, IF

NECESSARY:

NOTES TO PHYSICIAN: TREAT SYMPTOMATICALLY.

------5. FIRE-FIGHTING MEASURES ------

SUITABLE EXTINGUISHING MEDIA:

WATER FOG. DRY CHEMICAL. CARBON DIOXIDE (CO2). COOL CONTAINERS/TANKS WITH

WATER SPRAY.

UNSUITABLE EXTINGUISHING MEDIA:

KEEP AWAY FROM SOURCES OF IGNITION - NO SMOKING. COOL CONTAINERS / TANKS

WITH WATER SPRAY.

SPECIFIC HAZARDS ARISING FROM THE CHEMICAL:

EXTREMELY FLAMMABLE. KEEP PRODUCT AND EMPTY CONTAINER AWAY FROM HEAT AND

SOURCES OF IGNITION. RISK OF IGNITION. IN THE EVENT OF FIRE AND/OR

EXPLOSION DO NOT BREATHE FUMES. IN THE EVENT OF FIRE, COOL TANKS WITH WATER

SPRAY.

EXPLOSION DATA:

SENSITIVITY TO MECHANICAL IMPACT: NONE.

SENSITIVITY TO STATIC DISCHARGE: YES.

PROTECTIVE EQUIPMENT AND PRECAUTIONS FOR FIREFIGHTERS:

AS IN ANY FIRE, WEAR SELF-CONTAINED BREATHING APPARATUS PRESSURE-DEMAND,

MSHA/NIOSH (APPROVED OR EQUIVALENT) AND FULL PROTECTIVE GEAR.

------6. ACCIDENTAL RELEASE MEASURES ------

PERSONAL PRECAUTIONS, PROTECTIVE EQUIPMENT AND EMERGENCY PROCEDURES:

PERSONAL PRECAUTIONS:

EVACUATE PERSONNEL TO SAFE AREAS. ABSORB WITH SAND, CLAY, OR OTHER SUITABLE

MATERIAL. HARD SURFACES MAY BE MOPPED WITH WATER. AVOID CONTACT WITH SKIN,

EYES AND CLOTHING. REMOVE ALL SOURCES OF IGNITION. KEEP PEOPLE AWAY FROM

AND UPWIND OF SPILL/LEAK. TAKE PRECAUTIONARY MEASURES AGAINST STATIC

DISCHARGES. ALL EQUIPMENT USED WHEN HANDLING THE PRODUCT MUST BE GROUNDED.

STOP LEAK IF YOU CAN DO IT WITHOUT RISK.

ENVIRONMENTAL PRECAUTIONS:

ENVIRONMENTAL PRECAUTIONS:

BEWARE OF VAPORS ACCUMULATING TO FORM EXPLOSIVE CONCENTRATIONS. VAPORS CAN

ACCUMULATE IN LOW AREAS. PREVENT FURTHER LEAKAGE OR SPILLAGE IF SAFE TO DO

SO. DO NOT ALLOW MATERIAL TO CONTAMINATE GROUND WATER SYSTEM. PREVENT

PRODUCT FROM ENTERING DRAINS. LOCAL AUTHORITIES SHOULD BE ADVISED IF

SIGNIFICANT SPILLAGES CANNOT BE CONTAINED. DO NOT FLUSH INTO SURFACE

WATER OR SANITARY SEWER SYSTEM.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP:

METHODS FOR CONTAINMENT:

PREVENT FURTHER LEAKAGE OR SPILLAGE IF SAFE TO DO SO. ABSORB SPILL WITH

INERT MATERIAL (E.G. DRY SAND OR EARTH), THEN PLACE IN A CHEMICAL WASTE

CONTAINER.

METHODS FOR CLEANING UP:

PICK UP AND TRANSFER TO PROPERLY LABELED CONTAINERS. SOAK UP WITH INERT

ABSORBENT MATERIAL. CLEAN CONTAMINATED SURFACE THOROUGHLY. AFTER CLEANING,

FLUSH AWAY TRACES WITH WATER. GROUND AND BOND CONTAINERS WHEN TRANSFERRING

MATERIAL. PREVENT PRODUCT FROM ENTERING DRAINS. TAKE PRECAUTIONARY MEASURES

AGAINST STATIC DISCHARGES. SWEEP UP AND SHOVEL INTO SUITABLE CONTAINERS FOR

DISPOSAL.

------7. HANDLING AND STORAGE ------

PRECAUTIONS FOR SAFE HANDLING:

ADVICE ON SAFE HANDLING:

AVOID CONTACT WITH EYES. AVOID BREATHING VAPORS OR MISTS. HANDLE IN

ACCORDANCE WITH GOOD INDUSTRIAL HYGIENE AND SAFETY PRACTICE. TAKE

PRECAUTIONARY MEASURES AGAINST STATIC DISCHARGES. REMOVE AND WASH

CONTAMINATED CLOTHING BEFORE RE-USE.

CONDITIONS FOR SAFE STORAGE, INCLUDING ANY INCOMPATIBILITIES:

TECHNICAL MEASURES/STORAGE CONDITIONS:

KEEP CONTAINER TIGHTLY CLOSED IN A DRY AND WELL-VENTILATED PLACE. KEEP AWAY

FROM OPEN FLAMES, HOT SURFACES AND SOURCES OF IGNITION. KEEP IN PROPERLY

LABELED CONTAINERS. KEEP OUT OF THE REACH OF CHILDREN. STORE LOCKED UP.

INCOMPATIBLE PRODUCTS: STRONG ACIDS, ALKALIS, OR OXIDIZING AGENTS.

AEROSOL LEVEL: 3

------8. EXPOSURE CONTROLS/PERSONAL PROTECTION ------

CONTROL PARAMETERS:

EXPOSURE GUIDELINES:

CHEMICAL NAME   ACGIH TLV          OSHA PEL                  NIOSH IDLH

HEXANE          TWA: 50 PPM        TWA: 500 PPM              IDLH: 1100 PPM

64742-49-0      SKIN - POTENTIAL   TWA: 1800 MG/M3           TWA: 50 PPM

SIGNIFICANT        (VACATED) TWA: 50 PPM     TWA: 180 MG/M3

CONTRIBUTION TO    (VACATED) TWA: 180 MG/M3

OVERALL EXPOSURE

BY THE CUTANEOUS

ROUTE

PROPANE/        74-98-6:           74-98-6:                  74-98-6:

ISOBUTANE/      TWA: 1000 PPM      TWA: 1000 PPM             IDLH: 2100 PPM

N-BUTANE                           TWA: 1800 MG/M3           TWA: 1000 PPM

68476-86-8                                                   TWA: 1800 MG/M3

106-97-8:          (VACATED) TWA: 1000 PPM   106-97-8:

STEL: 1000 PPM                               TWA: 800 PPM

TWA: 1900 MG/M3

75-28-5:           (VACATED) TWA:            75-28-5:

STEL: 1000 PPM     1800 MG/M3                TWA: 800 PPM

TWA: 1900 MG/M3

106-97-8:

(VACATED) TWA: 800 PPM

(VACATED) TWA:

1900 MG/M3

PETROLEUM       TWA: 100 PPM       TWA: 500 PPM              IDLH:

DISTILLATES                                                  20000 MG/M3

8052-41-3

TWA: 2900 MG/M3           CEILING:

1800 MG/M3

15 MIN

(VACATED) TWA: 100 PPM    TWA: 350 MG/M3

(VACATED) TWA: 525 MG/M3

ACGIH: (AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS)

OSHA: (OCCUPATIONAL SAFETY & HEALTH ADMINISTRATION)

NIOSH IDLH: IMMEDIATELY DANGEROUS TO LIFE OR HEALTH

OTHER EXPOSURE GUIDELINES:

VACATED LIMITS REVOKED BY THE COURT OF APPEALS DECISION IN AFL-CIO V. OSHA,

965 F.2D 962 (11TH CIR., 1992).

EXPOSURE CONTROLS:

ENGINEERING MEASURES:

SHOWERS

EYEWASH STATIONS

VENTILATION SYSTEMS.

INDIVIDUAL PROTECTION MEASURES, SUCH AS PERSONAL PROTECTIVE EQUIPMENT:

EYE/FACE PROTECTION: SAFETY GLASSES WITH SIDE-SHIELDS.

SKIN AND BODY PROTECTION: CHEMICAL RESISTANT APRON. PROTECTIVE GLOVES.

RESPIRATORY PROTECTION:

IF EXPOSURE LIMITS ARE EXCEEDED OR IRRITATION IS EXPERIENCED, NIOSH/MSHA

APPROVED RESPIRATORY PROTECTION SHOULD BE WORN. POSITIVE-PRESSURE SUPPLIED

AIR RESPIRATORS MAY BE REQUIRED FOR HIGH AIRBORNE CONTAMINANT CONCENTRATIONS.

RESPIRATORY PROTECTION MUST BE PROVIDED IN ACCORDANCE WITH CURRENT LOCAL

REGULATIONS.

HYGIENE MEASURES:

HANDLE IN ACCORDANCE WITH GOOD INDUSTRIAL HYGIENE AND SAFETY PRACTICE.

------9. PHYSICAL AND CHEMICAL PROPERTIES ------

PHYSICAL AND CHEMICAL PROPERTIES:

PHYSICAL STATE: AEROSOL

APPEARANCE: OPAQUE

ODOR: SOLVENT

COLOR: DARK AMBER

ODOR THRESHOLD: NO INFORMATION AVAILABLE

PROPERTY                     VALUES                     REMARKS - METHODS

PH                           0

MELTING/FREEZING POINT       NO INFORMATION AVAILABLE

BOILING POINT/BOILING RANGE  NO INFORMATION AVAILABLE

FLASH POINT                  -97 DEG. C / -142 DEG. F   BASED ON PROPELLANT

EVAPORATION RATE             NO INFORMATION AVAILABLE

FLAMMABILITY (SOLID, GAS)    NO INFORMATION AVAILABLE

FLAMMABILITY LIMITS IN AIR:

UPPER FLAMMABILITY LIMIT     NO INFORMATION AVAILABLE

LOWER FLAMMABILITY LIMIT     NO INFORMATION AVAILABLE

VAPOR PRESSURE               NO INFORMATION AVAILABLE

VAPOR DENSITY                NO INFORMATION AVAILABLE

SPECIFIC GRAVITY             0.828

WATER SOLUBILITY             PRACTICALLY INSOLUBLE

PARTITION COEFFICIENT        NO INFORMATION AVAILABLE

N-OCTANOL/WATER

AUTOIGNITION TEMPERATURE     NO INFORMATION AVAILABLE   NOT APPLICABLE

DECOMPOSITION TEMPERATURE    NO INFORMATION AVAILABLE

VISCOSITY                    NO INFORMATION AVAILABLE

EXPLOSIVE PROPERTIES         NO INFORMATION AVAILABLE

OTHER INFORMATION:

VOC CONTENT(%)               83.58

MIR VALUE                    1.03

------10. STABILITY AND REACTIVITY ------

REACTIVITY: NO DATA AVAILABLE

CHEMICAL STABILITY: STABLE UNDER RECOMMENDED STORAGE CONDITIONS.

POSSIBILITY OF HAZARDOUS REACTIONS: NONE UNDER NORMAL PROCESSING.

CONDITIONS TO AVOID: EXTREMES OF TEMPERATURE AND DIRECT SUNLIGHT.

INCOMPATIBLE MATERIALS: STRONG ACIDS, ALKALIS, OR OXIDIZING AGENTS.

HAZARDOUS DECOMPOSITION PRODUCTS: NONE KNOWN BASED ON INFORMATION SUPPLIED.

------11. TOXICOLOGICAL INFORMATION ------

INFORMATION ON LIKELY ROUTES OF EXPOSURE:

PRODUCT INFORMATION:

INHALATION:

VAPORS MAY IRRITATE THROAT AND RESPIRATORY SYSTEM. MAY CAUSE DROWNSINESS

AND DIZZINESS BASED ON COMPONENTS. MAY CAUSE IRRITATION OF RESPIRATORY

TRACT. AVOID BREATHING VAPORS OR MISTS.

EYE CONTACT: IRRITATING TO EYES. AVOID CONTACT WITH EYES.

SKIN CONTACT:

IRRITATING TO SKIN. REPEATED EXPOSURE MAY CAUSE SKIN DRYNESS OR CRACKING.

PROLONGED SKIN CONTACT MAY DEFAT THE SKIN AND PRODUCE DERMATITIS. AVOID

CONTACT WITH SKIN.

INGESTION:

MAY BE HARMFUL OR FATAL IF SWALLOWED. ASPIRATION INTO THE LUNGS DURING

SWALLOWING MAY CAUSE SERIOUS LUNG DAMAGE WHICH MAY BE FATAL.

COMPONENT INFORMATION:

CHEMICAL NAME      LD50 ORAL        LD50 DERMAL       LC50 INHALATION

HEXANE             >5000 MG/KG      >3160 MG/KG       =73680 PPM (RAT) 4 H

64742-49-0         (RAT)            (RABBIT)

INFORMATION ON TOXICOLOGICAL EFFECTS:

SYMPTOMS: NO INFORMATION AVAILABLE.

DELAYED AND IMMEDIATE EFFECTS AS WELL AS CHRONIC EFFECTS FROM SHORT AND

LONG-TERM EXPOSURE:

SENSITIZATION: NO INFORMATION AVAILABLE.

GERM CELL MUTAGENICITY: NO INFORMATION AVAILABLE.

CARCINOGENICITY: THERE ARE NO KNOWN CARCINOGENIC CHEMICALS IN THIS PRODUCT.

REPRODUCTIVE TOXICITY:

PRODUCT IS OR CONTAINS A CHEMICAL WHICH IS A KNOWN OR SUSPECTED

REPRODUCTIVE HAZARD.

SPECIFIC TARGET ORGAN SYSTEMIC TOXICITY (SINGLE EXPOSURE):

MAY CAUSE RESPIRATORY IRRITATION. MAY CAUSE DROWSINESS AND DIZZINESS.

SPECIFIC TARGET ORGAN SYSTEMIC TOXICITY (REPEATED EXPOSURE):

MAY CAUSE DAMAGE TO ORGANS THROUGH PROLONGED OR REPEATED EXPOSURE.

CHRONIC TOXICITY: PROLONGED EXPOSURE MAY CAUSE CHRONIC EFFECTS.

TARGET ORGAN EFFECTS:

CENTRAL NERVOUS SYSTEM, EYES, KIDNEY, RESPIRATORY SYSTEM, SKIN.

NEUROLOGICAL EFFECTS:

INTENTIONAL MISUSE BY DELIBERATELY CONCENTRATING AND INHALING CONTENTS MAY

BE HARMFUL OR FATAL.

ASPIRATION HAZARD: MAY BE FATAL IF SWALLOWED AND ENTERS AIRWAYS.

NUMERICAL MEASURES OF TOXICITY - PRODUCT INFORMATION:

UNKNOWN ACUTE TOXICITY:

0% OF THE MIXTURE CONSISTS OF INGREDIENT(S) OF UNKNOWN TOXICITY

THE FOLLOWING VALUES ARE CALCULATED BASED ON CHAPTER 3.1 OF THE GHS DOCUMENT:

ATEMIX (ORAL): 11176 MG/KG

ATEMIX (DERMAL): 7063 MG/KG

------12. ECOLOGICAL INFORMATION ------

THIS PRODUCT CONTAINS A CHEMICAL WHICH IS LISTED AS A MARINE POLLUTANT

ACCORDING TO DOT.

ECOTOXICITY:

CHEMICAL NAME           TOXICITY  TOXICITY  TOXICITY TO      TOXICITY TO

TO ALGAE  TO FISH   MICROORGANISMS   DAPHNIA AND

OTHER AQUATIC

INVERTEBRATES

PROPANE/ISOBUTANE/

N-BUTANE

68476-86-8

PERSISTENCE AND DEGRADABILITY: NO INFORMATION AVAILABLE.

BIOACCUMULATION: NO INFORMATION AVAILABLE.

CHEMICAL NAME                                LOG POW

PROPANE/ISOBUTANE/N-BUTANE                   2.8

68476-86-8

OTHER ADVERSE EFFECTS: NO INFORMATION AVAILABLE

------13. DISPOSAL CONSIDERATIONS ------

WASTE TREATMENT:

WASTE DISPOSAL METHODS:

IT MUST UNDERGO SPECIAL TREATMENT, E.G. AT SUITABLE DISPOSAL SITE, TO

COMPLY WITH LOCAL REGULATIONS.

CONTAMINATED PACKAGING: DO NOT RE-USE EMPTY CONTAINERS.

------14. TRANSPORT INFORMATION ------

DOT:

GROUND CONSUMER COMMODITY ORM-D

OR

LIMITED QUANTITY

MARINE POLLUTANT:

THIS PRODUCT CONTAINS A CHEMICAL WHICH IS LISTED AS A MARINE POLLUTANT

ACCORDING TO DOT.

IATA: UN1950, AEROSOLS, FLAMMABLE, 2.1, LTD. QTY.

IMDG: UN1950, AEROSOLS, 2.1, LTD. QTY.

------15. REGULATORY INFORMATION ------

INTERNATIONAL INVENTORIES:

CHEMICAL NAME     TSCA  DSL/NDSL  EINECS/  ENCS    IECSC  KECL  PICCS  AICS

ELINCS

HEXANE            X     X         X        X       X      X     X      X

PROPANE/          X     X         X        NOT     X      X     X      X

ISOBUTANE/                                 LISTED

N-BUTANE

PETROLEUM         X     X         X        X       X      X     X      X

DISTILLATES

LEGEND:

TSCA: UNITED STATES TOXIC SUBSTANCES CONTROL ACT SECTION 8(B) INVENTORY

DSL/NDSL: CANADIAN DOMESTIC SUBSTANCES LIST/NON-DOMESTIC SUBSTANCES LIST

EINECS/ELINCS:

EUROPEAN INVENTORY OF EXISTING COMMERCIAL CHEMICAL SUBSTANCES/EU LIST OF

NOTIFIED CHEMICAL SUBSTANCES

ENCS: JAPAN EXISTING AND NEW CHEMICAL SUBSTANCES

CHINA: CHINA INVENTORY OF EXISTING CHEMICAL SUBSTANCES

KECL: KOREAN EXISTING AND EVALUATED CHEMICAL SUBSTANCES

PICCS: PHILIPPINES INVENTORY OF CHEMICALS AND CHEMICAL SUBSTANCES

AICS: AUSTRALIAN INVENTORY OF CHEMICAL SUBSTANCES

U.S. FEDERAL REGULATIONS:

SARA 313:

SECTION 313 OF TITLE III OF THE SUPERFUND AMENDMENTS AND REAUTHORIZATION

ACT OF 1986 (SARA). THIS PRODUCT DOES NOT CONTAIN ANY CHEMICALS WHICH ARE

SUBJECT TO THE REPORTING REQUIREMENTS OF THE ACT AND TITLE 40 OF THE CODE

OF FEDERAL REGULATIONS, PART 372.

SARA 311/312 HAZARD CATEGORIES:

ACUTE HEALTH HAZARD: YES

CHRONIC HEALTH HAZARD: YES

FIRE HAZARD: YES

SUDDEN RELEASE OF PRESSURE HAZARD: YES

REACTIVE HAZARD: NO

CLEAN WATER ACT:

THIS PRODUCT DOES NOT CONTAIN ANY SUBSTANCES REGULATED AS POLLUTANTS

PURSUANT TO THE CLEAN WATER ACT (40 CFR 122.21 AND 40 CFR 122.42).

CERCLA:

THIS MATERIAL, AS SUPPLIED, DOES NOT CONTAIN ANY SUBSTANCES REGULATED AS

HAZARDOUS SUBSTANCES UNDER THE COMPREHENSIVE ENVIRONMENTAL RESPONSE

COMPENSATION AND LIABILITY ACT (CERCLA) (40 CFR 302) OR THE SUPERFUND

AMENDMENTS AND REAUTHORIZATION ACT (SARA) (40 CFR 355). THERE MAY BE

SPECIFIC REPORTING REQUIREMENTS AT THE LOCAL, REGIONAL, OR STATE LEVEL

PERTAINING TO RELEASES OF THIS MATERIAL.

U.S. STATE REGULATIONS:

CALIFORNIA PROPOSITION 65:

THIS PRODUCT DOES NOT CONTAIN ANY KNOWN PROPOSITION 65 CHEMICALS.

U.S. STATE RIGHT-TO-KNOW REGULATIONS:

CHEMICAL NAME             NEW JERSEY    MASSACHUSETTS    PENNSYLVANIA

PETROLEUM DISTILLATES     X             X                X

8052-41-3

EPA PESTICIDE REGISTRATION NUMBER: NOT APPLICABLE

CANADA:

THIS PRODUCT HAS BEEN CLASSIFIED IN ACCORDANCE WITH THE HAZARD CRITERIA OF

THE CONTROLLED PRODUCTS REGULATIONS (CPR) AND THE MSDS CONTAINS ALL THE

INFORMATION REQUIRED BY THE CPR.

WHMIS HAZARD CLASS:

A: COMPRESSED GASES

B5: FLAMMABLE AEROSOL

D2B: TOXIC MATERIALS

------16. OTHER INFORMATION ------

NFPA:

HEALTH HAZARD                 2

FLAMMABILITY                  4

INSTABILITY                   0

PHYSICAL AND CHEMICAL HAZARDS

HMIS:

HEALTH HAZARD       2*

FLAMMABILITY        4

PHYSICAL HAZARD     1

PERSONAL PROTECTION B

CHRONIC HAZARD STAR LEGEND:

CHRONIC HEALTH HAZARD REPEATED OR PROLONGED EXPOSURE MAY CAUSE CENTRAL

NERVOUS SYSTEM DAMAGE

PREPARED BY:

AMERICAN JETWAY

34136 MYRTLE STREET

WAYNE, MI 48184

TEL.: 734-721-5930

ISSUING DATE: 16-APR-2015

REVISION DATE: 13-JUN-2015

REVISION NOTE: NO INFORMATION AVAILABLE

DISCLAIMER:

THE INFORMATION PROVIDED ON THIS SDS IS CORRECT TO THE BEST OF OUR

KNOWLEDGE, INFORMATION AND BELIEF AT THE DATE OF ITS PUBLICATION. THE

INFORMATION GIVEN IS DESIGNED ONLY AS A GUIDE FOR SAFE HANDLING, USE,

PROCESSING, STORAGE, TRANSPORTATION, DISPOSAL AND RELEASE AND IS NOT TO BE

CONSIDERED AS A WARRANTY OR QUALITY SPECIFICATION. THE INFORMATION RELATES

ONLY TO THE SPECIFIC MATERIAL DESIGNATED AND MAY NOT BE VALID FOR SUCH

MATERIAL USED IN COMBINATION WITH ANY OTHER MATERIAL OR IN ANY PROCESS,

UNLESS SPECIFIED IN THE TEXT.